PVC roll flooring needs a proper cleaning methods and maintenance to prolong its service life. 
Proper cleaning in two ways: dry cleaning, wet cleaning
1. Dry Cleaning: cleaning needs normal cleaning with a dry mop floor. Encountered stubborn stains can be removed with detergent. Steps are: get a clean mop, the "detergent" directly sprayed on the mop up feeling damp micro fiber per share can not be too wet. And then to spend the night stand-trailer in the ventilation, not just dealing with a good mop for immediate use. The treated dust mop can absorb and gently strong flick as long as the dust that off. Mop to clean, keep it clean. This dry method does not affect their daily work. 
2. Wet cleaning: water with a twist of dry mop the floor clean, and can add some specific floor cleaner in the water to enhance the cleaning effect. Clean the floor using this method, not carried out water, convenient save trouble. For stubborn stains, cloth stained with detergent is available. 
Maintenance Notice 
Prohibit the use of sharp objects on the floor for the impact, or scrape, cut, cut, shovel and so on. Prohibit the use of washing method of washing the stone ground floor. Carpet must be located at the entrance of the door, to reduce the wear and tear into the sediment on the floor